HELIDECK

INSPECTION REPORT

| Landing site Name | FSO TOPAZ |
| --- | --- |
| Owner/Operator | WORLD CARRIER SHIPMENT |
| Inspector(s) | OLISA KENECHUKWU |
| Operational Clearance (Day/Night) | DAY, NIGHT-MEDIVAC |
| D Value T Value | 21 12.8 |
| Date of Previous Inspection | NIL |
| Date of Current Inspection | 27TH July, 2026 |
| Date of Next Inspection | 26TH July, 2026 |

This report template and the contents herein are based on the provisions of UK CAA CAP 437 and the Nigerian NCAA NCARS

### INSTALLATION DETAILS

| Item | Comments |
| --- | --- |
| Installation Type Platform / FPSO / Semi-Sub / Seismic / NUI / MOPU / Lay Barge / Crane Barge / Support Vessel / Jack up / Other | FSO |
| Location of Inspection | EBOK FIELD |
| Area of operations | EBOK |
| Owners Name | ORIENTAL ENERGY RESOURCES |
| Owners Address |  |
| Name of contact |  |
| Job Position |  |
| Phone |  |
| E-mail |  |
| Helideck drawings – (a list of details which should be included at the end of this section) Drawing number: Date: Do the plans accurately show the detail that is required? If no, state omissions. | 1562-100-A-005 19th June 2026 YES |
| Helideck Height AMSL (feet) |  |
| Installation / Vessel identification Location of name (on all sides) of vessel Clearly visible Illumination | COMPLIANT |

### OFFSHORE HELIDECK INSPECTION CHECKLIST

| Manuals UK CAA CAP 437 Helicopter Procedures Manual detailing: - Normal duties - Emergency response - Training - Refueling Dangerous Goods ICAO/IATA DG Regulations Passenger Safety Briefing for aircraft type being used | NOT SIGHTED NOT COMPLIANT |
| --- | --- |

HELIDECK SURFACE

| Item | Comment |
| --- | --- |
| Colour: (If light grey or aluminum markings should be highlighted): Condition: (Good, fair, or poor): Cleanliness: (Good, fair, or poor): Level: | GREEN AND ALUMINIUM GOOD GOOD |
| Nonslip characteristics (estimated): High/Medium/Low/Nil: | HIGH |
| Last friction test: Date: Validity: one (1) year only For Exceptions: State Tested by whom: if less than 0.5mu, nets should be retained | APRIL 2026 |
| Gutter or raised curb? Around the entire perimeter: | GUTTER |
| Drainage: (Guttering clean?): | CLEAN |
| Deck sealed? (Deck sealed, no holes in deck for leaking/burning fuel) | SEALED |
| Aircraft Tie-down points: 3m, 5m, 7m? | COMPLIANT |

HELIDECK DIMENSIONS AND MARKINGS:

| Item | Comment |
| --- | --- |
| Overall Helideck Draw or cut and past the design of the helideck. |  |
| Overall Helideck Dimensions: Include run-off area if applicable: | 21D |
| Installation/rig/vessel name: State name painted on deck and position. Minimum 1.2 meters (preferably 2 meters) | TOPAZ COMPLIANT |
| Perimeter line: Should be white, 30cm wide. | COMPLIANT |
| Touchdown/Position Marking Circle: It should be yellow, 1 meter wide. State inner diameter (0.5D). | COMPLIANT |
| Chevron: Dimensions (0.79m x 0.10m), colour (black), angle (210o), location. | COMPLIANT |
| D value marking: Correct value? Colour, height, location. | 21D WHITE |
| "H": State size, colour, location, Swung? (4 x 3 x 0.75m,) | WHITE COMPLIANT |
| Max. Allowable Mass: Correct value, size & colour (0.9m, white), Helideck integrity Document Date: | 12.8 WHITE 29TH March 2026 |

| HELICOPTER TYPE | D VALUE | MAIN ROTOR DIAMETER | 0.05D | 0.12D | 0.21D | DISTANCE OF “H” FROM DECK EDGE (0.4D) | AIMING CIRCLE INSIDE DIAMETER (0.5D) | 0.62D | 0.83D | “T” VALUE | LANDING NET SIZE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EC135 | 12.19 | 10.2 | 0.61 | 1.46 | 2.51 | 4.8 | 6.1 | 7.55 | 10.12 | 2.9 | Not required |
| H160 | 15.68 | 13.4 | 0.78 | 1.88 | 3.29 | 6.27 | 7.84 | 9.72 | 13.01 | 6.0 | Medium |
| S76 | 16.00 | 13.40 | 0.80 | 1.92 | 3.36 | 6.40 | 8.00 | 9.92 | 13.28 | 5.3 | Medium |
| Agusta Westland AW139 | 16.66 | 13.8 | 0.83 | 2.0 | 3.50 | 6.66 | 8.33 | 10.3 | 13.82 | 6.4 | Medium |
| Agusta Westland AW189 | 18.0 | 14.60 | 0.90 | 2.16 | 3.78 | 7.2 | 9.0 | 11.16 | 14.94 | 8.6 | Super Medium |
| S92A | 20.88 | 17.17 | 1.04 | 2.51 | 4.38 | 8.35 | 10.44 | 12.95 | 17.33 | 12.6t | Large |

The above highlighted aircraft(s) is/are authorized to land on _FSO TOPAZ__________________________

| OBSTRUCTION ENVIRONMENT | Comment |
| --- | --- |
| 210o Sector obstruction(s): Location & height above helideck level. | COMPLIANT |
| 150o Sector obstruction(s): Location & height above helideck level. | COMPLIANT |
| 180o Sector, 5:1 Falling gradient obstruction(s): What & where. | COMPLIANT |
| Is combined operation being conducted? Duration. OFS compromised? Obstacle penetration of 5:1 falling gradient? Yellow prohibition marker placed on decks deemed unavailable? Any limitations on operation? LSSR update (If applicable) | COMPLIANT |

HELIDECK NET (IF APPLICABLE)

|  | Comment |
| --- | --- |
| Age/Condition: | NOT APPLICABLE |
| Tension: Lift not above 25 centimeters near the center. | NOT APPLICABLE |
| Location: Should cover Aiming Circle and not cover rig identification & max mass weight value. | NOT APPLICABLE |
| Material: Should be sisal, not polypropylene (Frictape type acceptable). | NOT APPLICABLE |
| Type: Threaded/Knotted/Inter-twined. | NOT APPLICABLE |
| Tie-down points: Every 1.5 meters. | NOT APPLICABLE |
| Method of Securing: Rope/Webbing/Ratchet Tensioners. | NOT APPLICABLE |
| Net size: 9 x 9m, 12 x 12m, 15 x 15m (covers the aiming circle not the name). | NOT APPLICABLE |
| Mesh size: 200mm/250mm. | NOT APPLICABLE |
| Rope diameter: 200mm. | NOT APPLICABLE |

| Perimeter Safety Net | Comment |
| --- | --- |
| Material & Condition: Wire Mesh, braided nylon, or Friction tape preferred (Polypropylene not recommended). Wire mesh, in satisfactory condition. | WIRE MESH SATISFACTORY |
| Width from deck edge: (1.5m). | COMPLIANT |
| Hammock effect: | UNKNOWN |
| Slope: Slope (10º), highest point above deck level. | COMPLIANT |
| Inspection & Test Schedule: (Note drop test is not to be carried out offshore) Regular condition inspection schedule offshore weekly recommended. | UNKNOWN |

PERIMETER LIGHTING

| Item | Comment |
| --- | --- |
| Colour & Serviceability: Note: Perimeter lights must be colored green, coincident to or adjacent with the white perimeter line. | GREEN COMPLIANT |
| Height above deck level (<25 centimeters) & distance apart: (3 meters) | COMPLIANT |
| Position: Do the lights delineate the safe landing area only, coincident with perimeter line, and visible for 360o ADL? | COMPLAINT |
| Power rating: (30 candelas green) | UNKNOWN |
| Limit of safe landing area: (Are red lights used to delineate the unsafe sectors?) | UNKNOWN |
| Emergency power supply Supply to the Perimeter and flood lights | COMPLAINT |
| Switching: controlled from the radio room or by HLO | UNKNOWN |

HELIDECK FLOOD LIGHTING

| Item | Comment |
| --- | --- |
| In Deck lighting system or Deck mounted Dazzle protection & power rating – state number and position. Do they dazzle the pilot during night operations? The recommended type of floodlights is of the deck mounted type. Spectral distribution: Do they provide adequate illumination of the center of the helideck? | COMPLAINT COMPLAINT COMPLAINT |

GENERAL LIGHTING

| Item | Comment |
| --- | --- |
| Installation/rig/vessel floodlighting: All floodlighting angled so as not to dazzle pilots | COMPLAINT |
| Structures 15 meter above deck level: Omnidirectional red lights every 10m above deck level with a minimum intensity of 10 candelas. To include all crane booms. | UNKNOWN |
| Highest point: Omni-directional light | COMPLAINT |

OBSTRUCTION MARKING AND LIGHTING

| Item | Comment |
| --- | --- |
| Obstruction lighting (it is acceptable for flare stacks and booms to be floodlit). | COMPLAINT |
| Colour of cranes and obstacles: (black/yellow or red/white preferred if close to the helideck or 150o Limited Obstacle Sector boundary). | WHITE |

STATUS LIGHT

| Item | Comment |
| --- | --- |
| A flashing Red Light that will signal to pilot that it is not safe to land. | COMPLAINT |
| Automatic switching. | UNKNOWN |
| Visible from all directions. | COMPLAINT |
| Connected to the emergency power supply. | UNKNOWN |
| Dual filament. | UNKNOWN |

ADDITIONAL EQUIPMENT

| Item | Comment |
| --- | --- |
| Chocks – number, type with a short connecting rope. (must be rubber, sandbags, minimum amount 6, 2 for each wheel). Short connecting rope to avoid looping over the S76 wheel axle. | COMPLIANT 1, PRESENT |
| Tie-down strops: minimum amount 6. Breaking strength 12.000 lbs. | COMPLIANT |
| Prohibited landing marker: 4 x 4-meter red flag with diagonal yellow cross. | NOT SIGHTED |
| Breathing apparatus: Minimum 2 sets with two reserve cylinders, stored in clearly marked and secure watertight cabinets or chests, container should be painted green and market with white letters, located close to the helideck, conduct operational check. | COMPLIANT |
| Windsock: Check number, position for best effect, and colour/condition. Must be lit for night operations. | COMPLIANT, LIT |
| Helicopter starter unit: Rectifier or battery trolley, 28V DC? Condition? | UNKNOWN |

TURBULENCE

| Item | Comment |
| --- | --- |
| Structures: items likely to cause turbulence. (jack up legs, derricks, cranes, other plat forms etc.) | NIL |
| Hot emissions likely to cause turbulence: flares, turbine exhaust. | EXHAUST PRODUCING SMOKE |
| Cold emissions: vents, blow down. | NIL |
| Air gap beneath helideck? | COMPLIANT |

ACCESS POINTS

| Item | Comment |
| --- | --- |
| Locations: minimum 2, upwind & downwind. Ensure that there are sufficient access points on small decks to ensure safe passage of personnel over the helideck. | 3, COMPLAINT |
| Handrails: collapsible/fixed/conspicuous. | COMPLIANT |
| Steps: clean and no trip/slip hazard. Good HSE practice | COMPLIANT |
| Safety Notices: available and clearly readable. | COMPLAINT |
| Control of non-helideck crew & passengers: procedures to prevent inadvertent access to helideck (e.g. chains). | UNKNOWN |

FIRE-FIGHTING EQUIPMENT

| Item | Comment |
| --- | --- |
| Foam monitors: quantity (minimum 2) and type (Aspirated/non-aspirated, manual/oscillating). | COMPLIANT, ASPIRATED |
| Fire main pressure: always on during helicopter operations? Good practice | COMPLIANT |
| Foam concentrate: type and quantity (for minimum amount of concentrate see table below). | 3% |
| Back-up concentrate must be 100%. | COMPLAINT |
| Foam Test Certificate: date done/due (must be tested annually). Note: Foam concentrates and produced foam to be tested. | APRIL, 2026 |
| Foam hand branches: quantity, type and condition: must have on/of control. Check proportioner setting. | 3 |
| Hydrant points: quantity and location. | 3, AT THE HELIDECK EXITS |
| Foam System Maintenance & Testing: Monitors must be functionally tested periodically for correct operation. Water test is acceptable. A competent person should test all parts of the foam production system annually. | COMPLIANT |
| Portable extinguishers (complementary media: Dry Powder & CO2) - Minimum number of extinguishers: - One 45kg dry powder (trolley type) at each access: - Dry Powder must be FOAM compatible type: - One 18kg CO2 with lance (trolley type): - Delivery rate 1.35 kg per second for Dry Powder & CO2 - Condition: last inspection date (minimum annual), particular attention to be paid to rubber hoses. | COMPLIANT |
| Back-up stocks of complementary media (100%) |  |

Foam requirement Table

| D Value | Area | AFFF 1% | AFFF 2% | AFFF 3% | AFFF 4% | AFFF 5% | AFFF 6% | Application rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (m) | (m²) | (Liters) | (Liters) | (Liters) | (Liters) | (Liters) | (Liters) | (Liters/min) |
| 13.68 | 147 | 44 | 88 | 132 | 176 | 220 | 264 | 882 |
| 16 | 201 | 60 | 120 | 180 | 241 | 301 | 361 | 1206 |
| 17 | 227 | 68 | 136 | 204 | 272 | 340 | 408 | 1362 |
| 18 | 254 | 76 | 152 | 229 | 305 | 381 | 458 | 1527 |
| 18.7 | 275 | 82 | 164 | 247 | 329 | 412 | 494 | 1648 |
| 19 | 283 | 85 | 170 | 255 | 340 | 425 | 510 | 1701 |
| 20 | 314 | 94 | 188 | 282 | 377 | 471 | 565 | 1885 |
| 21 | 346 | 103 | 207 | 311 | 415 | 519 | 623 | 2078 |
| 22.2 | 387 | 116 | 232 | 348 | 464 | 580 | 696 | 2322 |
| 22.8 | 408 | 122 | 245 | 367 | 490 | 612 | 735 | 2450 |
| 23 | 415 | 124 | 249 | 374 | 498 | 623 | 747 | 2493 |
| 30.18 | 715 | 214 | 429 | 643 | 858 | 1073 | 1287 | 4292 |

RESCUE EQUIPMENT

| ITEM DESCRIPTION | Quantity |
| --- | --- |
| Crash box (number of crash boxes (one by each access point recommended), accessible, weather-proof) | 1 |
| Adjustable wrench (one per crash box) x1 | 1 |
| Large rescue axe (one per crash box) x 1 | 1 |
| Bolt cutters (one per crash box) x 1 | 1 |
| Large crowbar (one per crash box) x 1 | 1 |
| Heavy duty hacksaw (one per crash box) x1 | 1 |
| Heavy duty blades (six per crash box) x 1 | 1 |
| Fire resistant blanket (one per crash box) x 1 | 1 |
| Side-cutting pliers (one per crash box) x 1 | 1 |
| Set of assorted screwdrivers (one per crash box) x 1 | 1 |
| Harness knife c/w (one knife for each crew member) x 1 | 1 |
| Man-Made Mineral Fibre (MMMF) Filter Masks (for each crew member) | 1 |
| Ladder: should be two sections of an aluminium ladder | UNKWN |
| Hook, grab or salving: one per crash box. X 1 | 1 |
| Fire resistant gloves (one pair for each crew member) | 1 |
| Lifeline (5 cm, 15m in length) x 1 | 1 |
| Power Cutting Tool. X 1 (Requires additional approved training by competent personnel) | 1 |

WATER RESCUE

| Item |  |
| --- | --- |
| Water rescue boat available in event of ditching (Type/location/available during all air operations). | COMPLIANT |
| Trained Rescue Personnel manning rescue boat (Number). | COMPLAINT |
| Procedure in place for water rescue. | UNKNOWN |

PROTECTIVE CLOTHING

| Item | Comment |
| --- | --- |
| Quantity, quality and condition of protective clothing: (available for each member of helideck crew) - Two piece suits (bunker coat and trousers) to meet standard EN469 or equivalent, OR - One piece suits (preferably orange) to meet standard EN 469 or equivalent - Helmet with full face visor to meet BS 3864 or equivalent - Gloves to meet standard EN 659 or equivalent - Boots to meet standard EN 345 or equivalent | COMPLIANT |
| HLO identifying waistcoat: contrasting colour to the rest of the helideck crew, flame retardant material, marked with the letters HLO in a reflective material on the front and back. | NOT COMPLIANT |
| Stowage for fire fighting equipment: Clothing should be hung and other equipment stored tidily in a quick access, clean and dry store near the helideck. | UNKNOWN |

RADIO OPERATOR & RADIO EQUIPMENT

| Item | Comment |
| --- | --- |
| Is the Radio Operator Competent and licensed with knowledge of and access to SOPs and Emergency procedures? | NOT SIGHTED |
| Do daily reports & departure messages contain all necessary information? (Position, weather, fuel state, POB, etc.) | NOT SIGHTED |
| For DP vessels, is there an “aircrew alert procedures” during helicopter rotors running on deck operations, when there is a significant change in the environmental conditions such as relative wind and speed, vessel heading and roll/pitch figures? (Status Lights, Radio, Visual Signals) | UNKNOWN |
| Portable multi-channel hand-held Radio (air band) including headset: number, type(s) and condition (a spare set is good practice). | COMPLIANT |
| Airband radios: type, condition and CAA approval (the second set is good practice to allow for redundancy and is required in busy areas so that the Air Traffic frequency remains free of Logistics chatter). | VHF |
| Portable VHF FM: (if applicable). | COMPLIANT |
| Radio Log – Record of helicopter movements, POB, endurance, etc. | NOT SIGHTED |

METEOROLOGICAL REPORTS & EQUIPMENT

| Item | Comment |
| --- | --- |
| Are weather reports in standard ICAO format? | COMPLIANT |
| Anemometer (fixed) – calibration date (annual). | COMPLIANT |
| Anemometer (hand) – calibration date (annual). |  |
| Wind direction: calibration date (annual), sensor in representative location? (Must be positioned in clear air.) | COMPLIANT |
| Air temperature – sensor in representative location? I.e. in the vicinity of the helideck. | COMPLIANT |
| Precision barometer/Twin Altimeters - height relative to helideck level, calibration date (annual) not sited inside pressurized compartment. | UNKNOWN |
| Visibility and cloud base equipment (if applicable): calibration date (annual). | COMPLIANT |
| Pitch, Roll and Heave equipment (if applicable): calibration date (annual) Note: Should be able to report the maximum peak levels of Pitch, Roll and Heave over the last 20 minutes. | COMPLIANT |
| Maintenance Schedule for instruments (especially automated systems): Daily/weekly checks by Observer plus scheduled maintenance and calibration in accordance with manufacturers’ instructions. Note: Observers should be trained to carry out daily/weekly checks. | UNKNOWN |

MISCELLANEOUS

| Item | Comment |
| --- | --- |
| Are pre-landing checks in accordance with a checklist & SOPs? | UNKNOWN |
| Is there dangerous goods reference material and documentation on board? | NOT SIGHTED |
| Helicopter emergency diagrams & safety posters – location? All types? | COMPLIANT |
| Helideck training exercises – written record - to be carried out at least once every month. | UNKNOWN |
| Video/CD player (state which) for showing passenger safety briefings - & suitable supervised area for briefing. Briefing must include all types expected to fly to the rig and reflect the configuration of the helicopter and the helicopter company. | FLASH DRIVE, COMPLIANT |
| Heavy-duty scales for baggage and freight: calibration date (annual). | COMPLIANT |
| Control of access to screened baggage/cargo: Documented baggage Processing procedures: | COMPLIANT |
| Stretcher – type and location. | NOT SEEN |

AVIATION FUEL SYSTEM

| Item | Comment |
| --- | --- |
| Type of System - Make: - Condition: - Pump running light: - Quick release connector on bonding lead | NOT APPLICABLE |
| Date of last Inspection - Authorized Fuel Inspector: - Date: - BHNL Fuel Inspector: - Date: | NOT APPLICABLE |
| On-Site Procedures - Weekly & Daily Checks - Provision of Shell test kits | NOT APPLICABLE |
| Documentation Retention of paperwork | NOT APPLICABLE |

HELIDECK CREW

| Item | Comment |
| --- | --- |
| HLO (Training to OPITO or equivalent standard) - HLO Course - Validity 2 years - Heli Team Members - Radio License | COMPLIANT |
| HDA (Training to OPITO or equivalent standard) - Heli Team member - Validity 3 years - DG by Air Awareness | COMPLAINT, 1 IS EXPIRED |
| Heli Admin - Radio License - . DG by Air Awareness | COMPLIANT |
| Manning Policy Current industry practice suggested the helideck crew consists of an HLO + 3. | COMPLIANT |
| Documentation and continuation training. Exercises and drills. | NOT SIGHTED |

DANGEROUS GOODS

| Item | Comment |
| --- | --- |
| Qualified personnel | UNKWOWN |
| Packaging | UNKNOWN |

RISK ASSESSMENT FOR HELICOPTER OPERATIONS TO HELIDECKS WHICH ARE SUB-1D

| Item | Comment |
| --- | --- |
| Obstacle clearance Potential for a reduction in the distance from helideck (TLOF) center to the limited obstacle sector (LOS) (denoting the origin of the 1st and 2nd segments). Is the TLOF surrounded by a 1D circle (FATO) free of any obstruction? | NOT APPLICABLE |
| Visual cues Are adequate visual cues available? - Identification of helideck location early on in the approach - Visual cues to help maintain the sight picture during approach - Visual cues on final approach to hover position - Visual cues for landing - Visual references on lift-off and hover | NOT APPLICABLE |
| Space for passengers Is there enough space available for passengers and crew to safely alight and embark the helicopter and to transit to and from the operating area safely? | NOT APPLICABLE |
| Helicopter Tie-down Is enough space available for securing helicopters, for the conduct of safe and efficient refueling operations (where provided) and for post-crash teams to provide effective fire and rescue intervention in the event of an incident or accident occurring? | NOT APPLICABLE |
| Touchdown/positioning inaccuracies Reduction in the margin built-in to allow for touchdown/positioning inaccuracies during landing. e.g limits on applied power for a combination of wind speed and direction. Are markings clean and free of contamination which may reduce a pilot’s ability to touch down accurately? | NOT APPLICABLE |
| Reduction in ‘helpful ground cushion effect from rotor downwash. The reduction in helpful ground cushion needs to be considered for each helicopter on a case-by-case basis, particularly when operating to a sub-1D helideck with a perforated surface i.e. some modern helideck designs incorporate a passive fire-retarding feature which allows unburned fuel to drain away through specially manufactured holes consisting in a drain-hole pattern over the surface of the load bearing area. | NOT APPLICABLE |

LANDING SITE INSPECTION REPORT

| Installation/Vessel Name | FSO TOPAZ |
| --- | --- |
| Owner/Operator | ORIENTAL |
| Inspector | OLISA KENECHUKWU |
| Operational Clearance (Day/Night) | DAY NIGHT (MEDIVAC ONLY) |
| D Value & T Value | D VALUE: 21M, T VALUE: 12.8T |
| Date of Previous Inspection | NIL |
| Date of Current Inspection | 27TH July 2026 |
| Date of Next Inspection (Maximum 2 years – Manned locations) (Maximum 2 years – NUI locations) | 26TH July 2028 |

Executive Summary:

Inspectors Executive Summary: I carried out a Helideck Inspection of the FSO TOPAZ on the 27th of July 2026. This inspection was conducted in accordance with the UK CAA CAP 437 and all other supporting documents.                              The Captain and HLO, were very instrumental during the inspection exercise and efforts are well appreciated.

The Helideck team showed a good level of competence in carrying out their duties. They don’t have an operational digitalized weather report system in place. The analogue weather systems were observed and are working. Also, there were no UPS power systems powering the importance systems as per CAP 437.

The HDA certificates were expired so recertification will be required, radio operator certificates were not sighted, I recommend that a UPS system be installed and an HMS system for weather be installed as well. Lastly their DGR manual was not sighted request for the manual has been made. Overall, it was a successful inspection, and the crew seemed to be willing to make these corrective adjustments.

And flight operations can commence at this helideck.

Inspector’s Executive Summary

| VHF and HAND HELD RADIOS | CALIBRATED WEIGHING SCALE |
| --- | --- |
| BRIEFING ROOM | WIND SOCK WITH ILLUMINTION |
| HELICOPTER CRASH BOX | ANEMOMETER AND INCLINOMETER |
| SEALED HELIDECK WITH AIR GAP | LIFEBOAT |
| TOP VIEW | AFFF FOAM AND WATER HOSE STATION |
| GOOD HELIDECK LIGHTING | PRIMETER SAFETY NET AND LIGHT |

Non-conformances are classified as:

- High Priority Critical Actions (these actions must be addressed before air operations can commence)

- Medium Priority Actions (these actions may impact continued operations and restrictions may be imposed)

- Low Priority Actions (these actions may lead to operating restrictions and must be addressed before a full approval is given)

## FORMAL LIST OF NON-COMPLIANCE

| # | Non-Compliance | Action Party | Priority (High/Med/Low) | Target Closure Date | Date Closed | How was Closure Verified |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Dangerous goods Manual not sighted | Oriental | Low | 30.SEPT.2026 |  |  |
| 2. | Normal and Emergency procedures not sighted | Oriental | Med | 30.SEPT.2026 |  |  |
| 3. | Training manual | Oriental | Med | 30.SEPT.2026 |  |  |
| 4. | Chocks 1, 6 required | Oriental | Low | 30.SEPT.2026 |  |  |
| 5. | Prohibited landing marker not sighted | Oriental | Low | 30.SEPT2026 |  |  |
| 6. | Ladder not sighted | Oriental | Med | 30.SEPT.2026 |  |  |
| 7. | HLO identifier on waistcoat not compliant | Oriental | Low | 30.SEPT.2026 |  |  |
| 8. | Radio operator license not sighted | Oriental | Low | 30.SEPT.2026 |  |  |
| 9. | HDA Opito certificate expired | Oriental | Med | 30.SEPT.2026 |  |  |

Recommendations to improve the helideck procedures, equipment or best practice. Not mandatory

## RECOMMENDATIONS

|  | Recommendation | Action party | Priority |  | Date closed | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | HELICOPTER SAFETY STICKERS SHOULD BE PLACED IN THE BRIEFING ROOM |  | LOW |  |  |  |

### HELIDECK LIMITATIONS LIST (HLL)

| Aircraft Type | Limitation Imposed | Reason for Limitation | Date Limitation Effective | Date Limitation Removed | Reason Limitation Removed |
| --- | --- | --- | --- | --- | --- |

### LANDING SITE INSPECTION ENDORSEMENT

#### NAME of LANDING SITE

#### NAME of BASE

#### DATE OF INSPECTION

| Helideck Inspector | Name: OLISA KENECHUKWU Signature: Date: 5TH September 2026 |
| --- | --- |
| Pilot-in-Charge / Chief Pilot | Name: Signature: Date: |
| Head of Flight Operations | Name: Signature: Date: |
| Quality and Safety Manager | Name: Signature: Date: |

Email Distribution List

- BHNL Pilots-in-Charge / Managing Pilots

- Head of Flight Operations

- Quality & Safety Manager

- Landing site coordinators

- Service delivery coordinator